# HomeBus Protocol Specification v0.1

## Overview

HomeBus is a distributed home automation network using:

- Full-duplex RS422

- 24V power distribution

- CH32X033-based nodes

- Gateway-based room segmentation

- Home Assistant as the system controller

The protocol is designed around:

- Physical Node IDs

- Logical Device Addresses

- OTA firmware deployment

- Distributed control

- Gateway routing

- Home Assistant integration

# Terminology

## Node

A physical HomeBus device.

Examples:

- Relay Node

- Fan Node

- Sensor Node

- Gateway Node

- Input Node

Each node has a unique:

Node ID

assigned using the 8-position DIP switch.

Node IDs are used for:

- Commissioning

- OTA updates

- Diagnostics

- Telemetry

Node IDs are NOT used for normal device control.

## Device

A logical controllable endpoint.

Examples:

- Bedroom Light

- Fan

- RGB Controller

- Spot Light

- Tuya Light

Each device has a logical:

Device Address

which is independent of the physical node.

This allows devices to be migrated between:

- HomeBus hardware

- Tuya devices

- Future integrations

without reconfiguring input nodes.

# Address Format

Device addresses are 32-bit.

0xRRTTDDDD

Where:

RR = Room ID       (8-bit)
TT = Device Type   (8-bit)
DDDD = Device ID   (16-bit)

Example:

0x02030005

Meaning:

Room 02
Type 03 (Spotlight)
Device 0005

# Reserved Addresses

0xFFFF

Home Assistant Controller

All telemetry is sent to this address.

# Node IDs

Node IDs are separate from Device Addresses.

Example:

Node ID = 42

This node may expose:

0x02030001
0x02030002
0x02030003
0x02030004

Each representing a separate relay output.

# Packet Types

HomeBus uses two primary packet classes.

## Control Packet

Used to control a logical device.

Structure:

Destination Address
Command
Payload Length
Payload
CRC16

Example:

Destination:
0x02030005

Command:
SET_STATE

Payload:
ON

The destination device executes the command.

The source is not transmitted.

The destination does not care who initiated the command.

## Telemetry Packet

Used by sensors and status-reporting nodes.

Structure:

Destination Address
Node ID
Sensor Type
Payload Length
Payload
CRC16

Example:

Destination:
0xFFFF

Node ID:
73

Sensor:
TEMPERATURE

Value:
26.4

Home Assistant resolves:

Node 73
+
Temperature

to the configured room and entity.

# Sensor Types

Suggested values:

0x01 Temperature
0x02 Humidity
0x03 Presence
0x04 Light Level
0x05 Current
0x06 Power
0x07 Voltage
0x08 Energy
0x09 Fan RPM
0x0A NTC Temperature

Additional types may be added later.

# Control Commands

Suggested values:

0x01 SET_STATE
0x02 TOGGLE
0x03 SET_PERCENT
0x04 INCREASE_PERCENT
0x05 DECREASE_PERCENT
0x06 SET_BRIGHTNESS
0x07 SET_COLOR
0x08 SET_PATTERN
0x09 IDENTIFY

# Gateway Operation

Gateways connect:

Backbone Bus

to

Room Bus

using two MAX491 transceivers.

Gateways maintain:

- Node table

- Device table

- Local cache

- Tuya mapping table

Gateways do NOT own device logic.

Gateways act as:

- Packet routers

- Protocol bridges

- OTA forwarders

# Device Resolution

Example:

0x02030005

may map to:

Tuya Device

today.

Later it may map to:

HomeBus Relay

without changing any switch configuration.

Input nodes only know:

Destination Address

and never know the implementation behind it.

# Sensor Reporting Policy

## Event Driven

Used for:

Presence
Door Sensors
Window Sensors
Switch Inputs
Encoders

Packets are transmitted only when state changes.

## Periodic

Used for:

Temperature
Humidity
Light Level
Power Monitoring

Packets are transmitted at configurable intervals.

Recommended:

30s - 300s

depending on sensor type.

# Fan Control

Fan nodes use fixed hardware mappings.

No pin-discovery commissioning is required.

Supported commands:

SET_PERCENT
INCREASE_PERCENT
DECREASE_PERCENT

Example:

+15%

The fan node maintains its own:

- Current speed

- Minimum stable speed

- Calibration data

The fan node is responsible for ensuring operation remains above stall threshold.

# Commissioning

Blank nodes boot into commissioning mode.

Node transmits:

Commission Request
Node ID
Capabilities

to Home Assistant.

If the Node ID already exists:

Firmware is restored automatically.

If the Node ID is unknown:

New commissioning process begins.

# OTA Updates

All nodes contain a permanent bootloader.

Recommended flash layout:

Bootloader

Slot A
Slot B

Update process:

Download new image
Verify CRC
Switch active slot
Reboot
Confirm health

Rollback occurs automatically if the new image fails.

# Reliability

All packets include:

CRC16

Invalid packets are discarded.

HomeBus uses:

Full Duplex RS422

throughout the network.

This allows:

- OTA updates

- Telemetry

- Control packets

to coexist without bus arbitration.